IEEE P802.11
Wireless LANs

| [A MAC Partial Proposal for IEEE 802.11s] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-07-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Rui Zhao | ComNets, RWTH Aachen University | M 225, Kopernikusstr. 16, 52074 Aachen, Germany | +49 241 80 27925 | rui@comnets.rwth-aachen.de |
| Bernhard Walke | ComNets, RWTH Aachen University | W 225, Kopernikusstr. 16, 52074 Aachen, Germany | +49 241 88 90320 | walke@comnets.rwth-aachen.de |
| Michael Einhaus | ComNets, RWTH Aachen University | M 324, Kopernikusstr. 16, 52074 Aachen, Germany | +49 241 80 23923 | ein@comnets.rwth-aachen.de |

Abstract

This document presents a MAC amendment for IEEE 802.11s by taking advantage of the multi-radio platform. A Wireless Distribution System (WDS) can be created like following: Mesh APs form a multi-hop network in a fully distributed manner on one frequency channel, while an 802.11 legacy or 802.11e station is associated with a nearby mesh AP of a Basic service Set (BSS) network and communicates with the mesh AP on another frequency channel. All features proposed here are fully compliant with the 802.11s PAR.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Additional Supporting Material

This section contains requirements for additional documentation that must be submitted with a proposal.  This is a template that must be filled in and included with a proposal submission.

| Number | Name | Definition | Coverage (Yes/No) |  | Notes | References |
| --- | --- | --- | --- | --- | --- | --- |
| AD1 | Reference submissions | A list of IEEE 802 submissions related to the proposal, both documents and presentations. |  | Yes |  | 11-05-0595-01-000s-mac-partial-proposal-Mesh-DCF.doc 11-05-0594-01-000s-mac-partial-proposal-Mesh-DCF.ppt |
| AD2 | Simulation and/or experimental methodology | Any proposal submission that includes simulation results must include a description of the simulation methodology used for mesh simulations. The simulation methodology documentation should provide enough information to, in principle, reproduce the simulation (e.g., including node positions, traffic and propagation model (including PHY assumptions), packet sizes, etc.). |  | Yes |  | 11-05-0594-01-000s-mac-partial-proposal-Mesh-DCF.ppt |

Coverage of Minimum Functional Requirements

This section contains a template for disclosure of coverage of minimum functional requirements with a proposal.   This template must be filled in and included with a proposal submission.

| Number | Category | Name | Coverage (Complete /Partial/ None) | Notes | References |
| --- | --- | --- | --- | --- | --- |
| FR1 | TOPO_RT_FWD | Mesh Topology Discovery | Partial |  | Section Error: Reference source not found |
| FR2 | TOPO_RT_FWD | Mesh Routing Protocol | Partial |  | Section Error: Reference source not found |
| FR3 | TOPO_RT_FWD | Extensible Mesh Routing Architecture | Partial |  | Section Error: Reference source not found |
| FR4 | TOPO_RT_FWD | Mesh Broadcast Data Delivery | Complete |  | Section Error: Reference source not found |
| FR5 | TOPO_RT_FWD | Mesh Unicast Data Delivery | Complete |  | Section Error: Reference source not found |
| FR6 | TOPO_RT_FWD | Support for Single and Multiple Radios | Complete |  | Section Error: Reference source not found |
| FR7 | TOPO_RT_FWD | Mesh Network Size | Partial |  | Section Error: Reference source not found |
| FR8 | SECURITY | Mesh Security | Partial |  | Section Error: Reference source not found |
| FR9 | MEAS | Radio-Aware Routing Metrics | Partial |  | Section Error: Reference source not found |
| FR10 | SERV_CMP | Backwards compatibility with legacy BSS and STA | Complete |  | Section Error: Reference source not found |
| FR11 | SERV_CMP | Use of WDS 4-Addr Frame or Extension | Complete |  | Section Error: Reference source not found |
| FR12 | DISC_ASSOC | Discovery and Association with a WLAN Mesh | Complete |  | Section Error: Reference source not found |
| FR13 | MMAC | Amendment to MAC with no PHY changes required | Complete |  | Section Error: Reference source not found |
| FR14 | INTRWRK | Compatibility with higher-layer protocols | Complete |  | Section Error: Reference source not found |

Table of Contents

1.	References	8

2.	Definitions	9

3.	Abbreviations and Acronyms	11

4.	Introduction	13

4.1.	Purpose	13

4.2.	General description	13

5.	Overview of MAC Services	16

5.1.	Protocol stack	16

5.2.	Overview of the MDCF	16

5.3.	Relay function	17

6.	Frame Formats in the MDCF	18

6.1.	General	18

6.2.	MACP Protocol Data Units (MPDUs)	18

6.2.1.	General frame format	18

6.2.2.1.	Frame control field	19

6.2.2.2.	QoS control field	19

6.2.2.	Management frames	20

6.2.2.1.	Beacon frame	20

6.2.3.	Control frames	20

6.2.3.1.	Link setup request (LinkSetupReq) frame	21

6.2.3.2.	Traffic channel reservation request (TCHReq) frame	21

6.2.4.	Data frames	21

6.3.	RLCP Protocol Data Units (RPDUs)	21

6.3.1.	General frame format	21

6.3.1.1.	Frame control field	22

6.3.2.	Link control frames	23

6.3.2.1.	Link setup request (LinkSetupReq) frame	23

6.3.2.2.	Link release request (LinkRelReq) frame	23

6.3.3.	Association control frames	24

6.3.4.	Radio resource control frames	24

6.3.4.1.	Traffic channel utilization request (TCHUtiReq) frame	24

6.3.4.2.	Traffic channel utilization respond (TCHUtiRes) frame	24

6.3.4.3.	Traffic channel reservation request (TCHReq) frame	24

6.3.5.	AM Data frames	25

6.3.5.1.	Request frame for adjusting ARQ parameters (ARQParReq)	25

6.3.5.2.	Respond frame for adjusting ARQ parameters (ARQParRes)	25

6.3.5.3.	ARQ polling frame	25

6.3.5.4.	ARQ data frame	25

6.3.5.5.	ARQ status report (ARQStatusRep) frame	25

6.3.6.	UM data frames	26

7.	MDCF description	27

7.1.	Architecture of MDCF	27

7.1.1.	Media Access Control Protocol (MACP)	27

7.1.2.	Radio Link Control Protocol (RLCP)	28

7.2.	MACP	29

7.2.1.	TDMA frame and energy signals	29

7.2.2.	Prioritized access	30

7.2.2.1.	Contention process	30

7.2.2.2.	PP Contention levels (PPCLs)	31

7.2.2.3.	Fairness access	32

7.2.3.	Link setup and TCH reservation	32

7.2.4.	Transmission and On-demand-TDD	33

7.2.5.	Calming down hidden stations	34

7.2.6.	Eliminating Exposed Station problem	36

7.2.7.	Packet multiplexing	36

7.2.8.	Multi-hop operation	37

7.2.9.	Broadcast and multicast	37

7.2.10.	TCH control	37

7.2.10.1.	Hang-on release	37

7.2.10.2.	Valid transmission time (VTT)	38

7.2.10.3.	Forced release of TCH	38

7.2.10.4.	Adaptation of TCH number for a link	38

7.2.10.5.	MACP Protocol Data Unit (MPDU) trains	39

7.2.11.	Synchronization	40

7.2.11.1.	General	40

7.2.11.2.	Maintaining synchronization	41

7.2.11.3.	The MTSF finite state machine	41

7.2.11.4.	MTSF timers	43

7.2.11.5.	On reception of a valid beacon	44

7.2.11.6.	Multi-hop support	46

7.3.	RLCP	46

7.3.1.	Service modes	46

7.3.1.1.	UM	46

7.3.1.2.	AM	47

7.3.2.	Error control and flow control (informative)	47

7.3.3.	RLCP transmission processes	48

7.3.3.1.	Virtual Link setup	48

7.3.3.2.	Data delivery	49

7.3.3.3.	TCH reservation request	50

7.3.4.	Allocation of service entities	50

7.3.5.	Adaptation of ARQ parameters during transmission	50

7.3.6.	Radio Resource Control	50

7.3.6.1.	Call Admission Control (CAC)	50

7.3.6.2.	Adjusting of traffic channels for use	52

8.	Compatible issues	53

8.1.	Physical layer (PHY)	53

8.2.	Security	53

8.3.	Mesh topology discovery	53

8.4.	Routing	53

8.5.	Mesh network size	53

9.	Acknowledgements	54

List of Figures

Figure 1: An example of WDS	13

Figure 2: The Data Link Layer model in a Mesh AP	14

Figure 3: Mapping of IEEE 802 Reference Model to scope of the proposal.	16

Figure 4: Relationships of various PDUs.	18

Figure 5: MPDU frame format.	19

Figure 6: Frame control field in the MPDU	19

Figure 7: Beacon frame format	20

Figure 8: Control frame format	20

Figure 9: Format of Data Frame Type 1	21

Figure 10: Format of Data Frame Type 2	21

Figure 11: Format of Data Frame Type 3	21

Figure 12: RPDU frame format	22

Figure 13: Frame control field in the RPDU	22

Figure 14: Format of request frame for ARQ parameter changing.	25

Figure 15: Format of ARQ data frame.	25

Figure 16: Format of ARQ status report frame.	26

Figure 17: Format of UM Data frame.	26

Figure 18: The architecture of MDCF	27

Figure 19: TDMA frame and energy signals	29

Figure 20: ACH structure.	30

Figure 21: An example of contending for an access. STA S1, S2 and S3 are in the transmission range of one another.	31

Figure 22: Process for link setup and TCH reservation.	32

Figure 23: An example of calming down hidden station nearby transmission pairs by transmitting BESes.	33

Figure 24: An example of transmission process and on-demand-TDD.	33

Figure 25: Calming down hidden station in MDCF mesh networks.	35

Figure 26: An example of the packet multiplexing between two mesh APs.	36

Figure 27: Multi-hop forwarding might take place in a same TDMA frame	37

Figure 28: Release of a TCH after the expiration of the set hang-on time. The hang-on time in the example is 2 TDMA frames.	38

Figure 29: An example of adaptation of TCH number for a link. The hang-on time is 1 TDMA frame and the maximum allowed number of TCHs for the link is 3.	39

Figure 30: An example of transmitting as MPDU trains.	39

Figure 31: Finite state machine of the MTSF.	42

Figure 32: Time skew in TDMA operation	44

Figure 33: Data delivery between two UM entities over the radio interface	47

Figure 34: Data delivery between two AM entities over the radio interface	47

Figure 35: RLCP Transmission processes	49

List of Tables

Table 1: Type and Subtype combinations	19

Table 2: QoS Control fields	20

Table 3: Beacon frame body	20

Table 4: Type and Subtype combinations	22

Table 5: Frame body of the connection request frame	23

Table 6: Frame body of the disconnection request frame	23

Table 7: Frame body of connection request frame	24

Table 8: Frame body of the traffic channel reservation frame	24

Table 9: Contention levels in the MDCF.	31

Table 10: The states used in the MTSF state machine.	42

Table 11: Timers in the MTSF.	43

- References

- IEEE 802 11-04/54r2, PAR for IEEE 802.11s ESS Mesh.

- IEEE 802 11-04/56r1, Five Criteria for IEEE 802.11s ESS Mesh.

- IEEE Std. 802.11, Wireless LAN Media Access Control (MAC) and Physical Layer (PHY) specification, 1999.

- IEEE 802.11e/D13.0, Medium Access Control (MAC) Quality of Service (QoS) Enhancements, Jan. 2005

- IEEE 802 11-04/662r11, TGs Usage Models.

- S. Mangold, S. Choi, G. Hiertz, O. Klein and B. Walke, "Analysis of IEEE 802.11 for QoS Support in Wireless LANs," In IEEE Wireless Communications, Vol. 10, pp. 2-12, Dec. 2003

- D. Chen, S. Garg,, M. Kappes, and K. S. Trivedi, "Supporting VoIP traffic in IEEE 802.11 WLAN with enhanced medium access control (MAC) for quality of service", www.research.avayalabs.com/techreport/ALR-2002-025-paper.pdf.

- F. A. Tobagi and L. Kleinrock, “Packet Switching in Radio Channels: Part II—The Hidden Terminal Problem in Carrier Sense Multiple-access and the Busy-tone Solution,” IEEE Trans. Commun., Vol. COM-23, pp. 1417–1433, Dec. 1975.

- IEEE 802.11a: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications: High-speed Physical Layer in the 5 GHZ Band, 1999.

- RFC 2598 - An Expedited Forwarding PHB, June 1999.

- B. Williams and T. Camp, “Comparison of broadcasting techniques for mobile ad hoc networks.” Proc. ACM Symposium on Mobile Ad Hoc Networking & Computing (MOBIHOC 2002), pp.194–205, 2002.

- Definitions

Acknowledged mode (AM): The service mode which provided by the RLCP for reliable unicast transmissions.

Access channel (ACH): Time slots used to contend for the channel access by using AESes.

Access energy signal (AES): The energy signals used to implement the prioritized access mechanism in the MDCF.

Association: The mobile station used to establish a station membership in a BSS network.

Busy energy signal (BES): The energy signals used to notify the occupancy of traffic channels.

Call admission control (CAC): An algorithm to ensure the admittance of a new flow into a resource constrained network does not violate parameterized service commitments made by the network to admitted flows.

Disassociation: The mobile station removing an existing association.

Echo channel (ECH): Time slots used to transmit BESes.

Energy signal: A short period of busy signal transmitted over the wireless medium by a transmitter. Energy signals are classed into two types according to the time slots in which they are used: AESes in the ACH while BESes in ECHs.

Fragmentation: In the MDCF, the process of partitioning of a LLC PDU into a sequence of smaller RPDUs prior to transmission. The process of recombining a set of fragment RPDUs into a LLC PDU is known as defragmentation.

Frame: The format of aggregated bits from a protocol entity that are transmitted together in time.

Hidden station: A station whose transmissions cannot be detected using carrier sense by a second station but whose transmissions interfere with transmissions from the second station to a third station,

MACP protocol data unit (MPDU): The units of data exchanged between two peer MACP entities in MDCF networks, using the services of the physical layer.

Mesh distributed coordination function (MDCF): A class of distributed function used in each mesh AP to form a multi-hop network based on the TDMA/TDD technology.

Link: A physical path consisting of exactly one traversal of the wireless medium that is used to transfer RPDUs between two stations.

Virtual Link identifier (V-Link ID): Each service entity in the RLCP entity is assigned with a virtual link identification number. A V-Link ID should be unique between a transmission pair. In a mesh AP, RPDUs from service entities with different V-Link IDs but a same destination shall be multiplexed on the reserved TCHs between the mesh AP and its transmission partner.

Protocol Data Unit (PDU): The unit of data exchanged between peer entities.

Packet: The format of aggregated bits that are transmitted together in time across the physical medium,

Piggyback: The overloading of a frame of type data with an acknowledgement of previously received RPDUs to the station to which the frame is directed.

Quality of Service (QoS): A collective measure of the level of service delivered between mobile stations. Quality of Service is characterized by basic performance criteria, including availability, drop rate, throughput, packet delay and jitter and connection setup time.

RLCP protocol data unit (RPDU): The units of data exchanged between two peer RLCP entities in MDCF networks, using the services of MACP.

Time division duplexing (TDD): A transmission method that uses only one time slot both for transmitting and receiving.

Time division multiple access (TDMA): Dividing a radio frequency into time slots and then allocating slots to multiple links. In this way, a single frequency can support multiple, simultaneous data channels.

Traffic channel (TCH): Time slots used to transmit data packets.

Traffic identifier (TID): Any of identifiers usable by higher-layer entities to distinguish MPDUs to support QoS within the MACP in the MDCF.

Unacknowledged mode (UM): The service mode provided by the RLCP for connectionless unicast, multicast and broadcast transmissions.

Wireless medium (WM): The medium used to implement the transfer of PDUs between peer PHY entities of an WLAN.

- Abbreviations and Acronyms

ACH 	Access Channel

AES	Access-E-Signal

AGC	Automatic Gain Control

AM 	Acknowledged Mode

AP 	Access Point

ARQ	Automatic Request

BES	Busy-E-Signal

BSS	Basic Service Set

BSSID	Basic Service Set identification

CAC 	Call Admission Control

CRC 	Cyclic Redundancy Code

DA	Destination Address

DCF 	Distributed Coordination Function

DiffServ 	Differentiated Services

DL	Data Link

DVB 	Double Value Busy-E-Signal

ECH 	Echo Channel

EDCA	Enhanced Distributed Channel Access

EF 	Expedited Forwarding

ESS	Extended Service Set

HCF	Hybrid Coordination Function

FA 	Fair Access

FCS	Frame Check Sequence

FTP 	File Transmission Protocol

V-Link ID	Virtual Link identification

LLC	Logic Link Control

LSB	Least Significant Bit

MAC 	Media Access Control

MACP	Media Access Control Protocol

MDCF	Mesh Distributed Coordination Function

MPDU	MACP Protocol Data Unit

MSB	most Significant Bit

MTSF	MDCF time synchronization Function

nQSTA	non-QoS Station

PDU	Protocol Data Unit

PHB 	Per Hop Behaviors

PHY  	Physical layer

PLR	Packet Loss Rate

PPM	parts per million

QoS	Quality of Service

QTS	QoS-related Traffic Specification

QSTA	QoS station

RA	Receiver Address

RLCP 	Radio Link Control Protocol

RPDUs	RLC PDUs

SA	Source Address

SR-ARQ	Selective Repeat Automatic Request

STA	Station

SVB 	Single Value Busy-E-Signal

TA	Transmitter Address

TC	Traffic Category

TCH 	Traffic Channel

TCP		Transmission Control Protocol

TDD 		Time Division Duplex

TDMA 		Time Division Multiple Access

UM 	Unacknowledged Mode

VoIP	Voice over IP

VTT 	Valid Transmission Time

WDS	Wireless Distribution System

WM	Wireless Medium

WLAN 		Wireless LAN

- Availability

- Due to IEEE copyright policies the document is not available at ComNets. Please download the document at:

- http://www.802wirelessworld.com